Ultrasonic Mount Assembly

- The following parts shall be CNC machined, us_mount_pn.prt and us_bottom_plate_pn.prt, model files and PDF drawings attached.

- The critical features on these parts are highlighted in yellow on the PDF prints. These features are critical to allow for relative motion between the plates.

- The purchased shaft, McMaster part number 1555T121, shall be modified per print us_1555T121_Linear_Shaft_Draw, to have a 10-24 tapped hole in one end at 0.500” deep. See attached PDF print of modified shaft.

- Two press-fit bushings, McMaster part number 3671N137, shall be pressed into the critical holes identified in print us_mount_pn. These bushings provide a bearing surface for the linear shaft.

- The remainder of the components, the shaft clamping collars and stepper motors, shall be assembled per the attached assembly print.